# LINK AGGREGATION, LACP & PAgP

Table of Contents

1.	Configure and verify (Layer2/Layer3) EtherChannel (LACP)	1

Reasons to use EtherChannel	2

Layer 2 EtherChannel	2

Layer 3 EtherChannel	2

Benefits of EtherChannel	3

Requirements to make EtherChannel:	3

Relationship of Physical Ports, Logical Port Channels, and Channel Groups:	4

EtherChannel Load Balancing Algorithm:	4

Types of EtherChannel	4

1. Static EtherChannel	4

Lab on EtherChannel	5

Creating bundle	5

Dynamic EtherChannel	7

2. Dynamic EtherChannle	7

LACP	7

PAGP	7

# Configure and verify (Layer2/Layer3) EtherChannel (LACP)

Figure 1: STP without EtherChannel

Why is one link blocked between the switches?

By default, cisco switches have a layer 2 loop prevention protocol known as Spanning-Tree Protocol (STP). Therefore, physically interconnecting switches, as shown in the previous diagram, will cause STP to automatically block one of the interfaces.

Bit of background

This is where EtherChannels come in to save us once more. An EtherChannel allows us to combine multiple physical ports on a switch to create a single logical interface. Therefore, the EtherChannel will carry the total bandwidth of all physical ports combined.

Definition

EtherChannel is a technology that bundles multiple physical Ethernet links into one logical link. It provides increased bandwidth, redundancy, and load balancing.

## Reasons to use EtherChannel

Increase bandwidth

Redundancy

Load balancing

STP treats it as one logical link

Faster convergence after a failure

Solution provided by EtherChannel

Single-link bottleneck

Single point of failure

STP blocking redundant links

## Layer 2 EtherChannel

There are Layer 2 EtherChannel as well Layer 3 EtherChannel. Layer 2EtherChannel is a logical interface that bundles multiple physical switch ports into a single Layer 2 link. It operates as a switchport, carries Ethernet frames and VLAN traffic, can be configured as either an access port or a trunk port, and does not have an IP address because it performs switching rather than routing.

What must match before an EtherChannel Forms

Speed, Duplex, VLAN

## Layer 3 EtherChannel

Layer 3 EtherChannel is a logical inter

Figure 1: STP with EtherChannel

IT is used to aggregate multiple physical link into a logical link called port channel.

## Benefits of EtherChannel

EtherChannel provides the following benefits in an enterprise network:

- Rather than configuring individual interfaces, the configurations can be done directly on the EtherChannel interface, rather than the physical ports.

- Implementing EtherChannels on a network can assit with load balancing and the link aggregation of traffic between switches.

- EtherChannels use the existing physical interfaces on a switch; therefore, we do not need to install additional modules.

## Requirements to make EtherChannel:

Requirements:

Speed must match.

Duplex must match.

Ethernet IEEE standard must match.

Trunk encapsulation protocol must match

Trunk allowed vlan list must match.

Native vlan must match.

Prunning eligibility list must match (unwanted broadcast on both sides should be blocked).

## Relationship of Physical Ports, Logical Port Channels, and Channel Groups:

## EtherChannel Load Balancing Algorithm:

- Src mac address (bydefault running)

- dst mac address

- src and dst mac address

- src ip add

- ds tip add.

- Src and ds tip add

- Src port number

- Dst port no.

- Src and dst port number

## Types of EtherChannel

### 1. Static EtherChannel

Static EtherChannel:

Sw(config-rang-if)#channel-group 1 mode on

Static etherchannel supports maximum 8 links aggregation.

Mode=ON

NOTE: When we configure an EtherChannel in the ON mode, no negotiations take place. The switch forces all compatible ports to become active in the EtherChannel. The other end of the channel (on the other switch) must also be configured in the on mode; otherwise, packet loss can occur.

## Lab on EtherChannel

Here, switch 1 is root switch as all the ports are  Dp or Fwd port

Fa0/3 Desg FWD 19 128.3 P2p

Fa0/4 Desg FWD 19 128.4 P2p

Fa0/1 Desg FWD 19 128.1 P2p

Fa0/2 Desg FWD 19 128.2 P2p

Switch1#

--------------------------------

Fa0/1 Root FWD 19 128.1 P2p

Fa0/2 Altn BLK 19 128.2 P2p

Fa0/3 Altn BLK 19 128.3 P2p

Fa0/4 Altn BLK 19 128.4 P2p

Switch0

### Creating bundle

If we are configuring new device, we will shut them first and we configure them but if we are working on existing/working device, we won’t shut as that is helping in communication.

Int range fa0/1-4

!switchport trunk encap dot1q

Switchport mode trunk

Channel-group 1 mode on

No shutdown

Exit

Int po1

Swi trunk encap dot1q

Sw mo trunk

no sh

this above case usually happens itself. Incase if it doen’t happens then we should run above commands.

Sh interfaces trunk

Sh etherchannel summary

Sh interfaces fa0/1

On another switch

exit

interface range fa0/1-4

swithcport mode trunk

switchport mode trunk

channel-group 1

mode on

channel-group 1 mode on

no shut

exit

do sh hist

Switch0(config)#

Conclusion

We used 4 ports and bundled them.

## Dynamic EtherChannel

Here, we added another switch with 3 port assigned.

Now we will use static etherchannel on here again as above.

interface range fa0/5-7

swithchport mode trunk

!Switchport trunk encap dot1q

switchport mode trunk

switchport nonegotiate

channel-group 2 mode on

no shut

exit

do sh hist

Switch2(config)#

ON means static etherchannel and it is mode rather than protocol.

### 2. Dynamic EtherChannle

### LACP

### PAGP

LACP (open standard)

- Active ( both side active is correct)

- Passive ( both side passive is incorrect)

PAgP (cisco)

- Auto (both side auto is wrong)

- Desirable ( both side desirable is correct)

We can’t mix protocol between LACP and PAgP but we can mix between their own working protocol.

Tips: it is always the best to use static etnerchannel in server.

Switch0(config-if-range)#do sh hist

interface fa0/22-24

interface range fa0/22-24

switchport mode trunk

switchport nonegotiate

no shut

channel-protocol pagp

channel-group mode desirable

channel-group 3 mode desirable

no shut

do sh hist

interface range fa0/21, fa0/23-24

switchport mode trunk

switchport nonegotiate

no shutdown

channel-group 3 mode pagp

channel-protocol pagp

channel-group 3 mode auto

no shut

exit

do sh hist

Switch2(config)#

# Layer 3 Etherchannel

Ethernet port means LAN